2. Demonstrate the use of digital write () function by blinking LED

void setup() {

pinMode(LED_BUILTIN, OUTPUT);   // Set built-in LED pin as output

}

void loop() {

digitalWrite(LED_BUILTIN, HIGH);  // Turn LED ON

delay(2000);                     // Wait for 2 seconds

digitalWrite(LED_BUILTIN, LOW);   // Turn LED OFF

delay(3000);                     // Wait for 3 seconds

}

Connection:

Connect LED long leg (Anode) → Pin 13

Connect LED short leg (Cathode) → Resistor (220Ω)

Connect other end of resistor → GND

3. Demonstrate the use of analogy write function

int led = 9;          // LED connected to PWM pin 9

int brightness = 0;   // Initial brightness

int fadeAmount = 5;   // Amount to increase/decrease brightness

void setup() {

pinMode(led, OUTPUT);   // Set LED pin as output

}

void loop() {

analogWrite(led, brightness);   // Set brightness

brightness = brightness + fadeAmount;  // Change brightness

// Reverse direction at limits

if (brightness <= 0 || brightness >= 255) {

fadeAmount = -fadeAmount;

}

delay(30);   // Small delay for smooth fading

}

Connection

Connect LED long leg (Anode) → Pin 9 (PWM pin)

Connect LED short leg (Cathode) → 220Ω Resistor

Connect other end of resistor → GND

2. Measure temperature of an environment using LM 35 with Arduino

Code;

const int lm35_pin = A1;   // LM35 output pin

void setup() {

Serial.begin(9600);

}

void loop() {

int temp_adc_val;

float temp_val;

temp_adc_val = analogRead(lm35_pin);   // Read temperature

temp_val = (temp_adc_val * 4.88);      // Convert ADC value to millivolts

temp_val = (temp_val / 10);            // LM35 gives 10mV per °C

Serial.print("Temperature = ");

Serial.print(temp_val);

Serial.print(" Degree Celsius\n");

delay(1000);

}

Connection

lM35 Temperature sensor to Arduino

Out pinA1

GNDGND

VCC5V

3. Implement push button to control buzzer with Arduino

Code:

int buzzerPin = 8;

int buttonPin = 7;

void setup() {

pinMode(buzzerPin, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);

}

void loop() {

int buttonState = digitalRead(buttonPin);

if (buttonState == LOW) {

digitalWrite(buzzerPin, HIGH);   // Turn buzzer ON

}

if (buttonState == HIGH) {

digitalWrite(buzzerPin, LOW);    // Turn buzzer OFF

}

}

Connection Buzzer to Arduino

Buzzer + (long leg) → Pin 8

Buzzer – (short leg) → GND

Connection Push Button to Arduino

One side of button → Pin 7

Other side of button → GND

4. Demonstrate the keypad working with an Arduino

Code:

#include <Keypad.h>

const byte ROWS = 4; // four rows

const byte COLS = 4; // four columns

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[ROWS] = {9, 8, 7, 6}; // connect to row pinouts of keypad

byte colPins[COLS] = {5, 4, 3, 2}; // connect to column pinouts of keypad

// Create an object of keypad

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

void setup() {

Serial.begin(9600);

}

void loop() {

char key = keypad.getKey(); // Read the key

if (key) { // If key pressed

Serial.print("Key pressed: ");

Serial.println(key);

}

}

Connection:

Keypad  Arduino

Left to right connection

1-D8

2-D7

3-D6

4-D5

5-D4

6-D3

7-D2

8-D1

5. Detect motion using PIR sensor

Code:

int led = 13;       // LED pin

int sensor = 2;     // PIR sensor pin

int state = LOW;    // Current state

int val = 0;        // Sensor reading

void setup() {

pinMode(led, OUTPUT);

pinMode(sensor, INPUT);

Serial.begin(9600);

}

void loop() {

val = digitalRead(sensor);   // Read PIR sensor value

if (val == HIGH) {           // If motion detected

digitalWrite(led, HIGH);

delay(100);

if (state == LOW) {

Serial.println("Motion detected!");

state = HIGH;            // Update state

}

}

else {

digitalWrite(led, LOW);

delay(200);

if (state == HIGH) {

Serial.println("Motion stopped!");

state = LOW;             // Update state

}

}

}

PIR to Arduino

VCC → 5V (Arduino)

OUT → Pin 2 (Arduino)

GND → GND (Arduino)

LED Connection to Arduino

(+) legD13

(-) legGND